ТЕХНИЧЕСКОЕ ОПИСАНИЕ ТОВАРОВ «ТРУБНЫЕ МАТЕРИАЛЫ И ОБОРУДОВАНИЕ»/ GOODS’ TECHNICAL DESCRIPTION “PIPING MATERIALS AND EQUIPMENT”

Позиция 1 - УСТЬЕВОЕ ОБОРУДОВАНИЕ

Комплектация устьевого оборудования

Устьевое оборудование состоит из многоступенчатого корпуса.

Тип A – обсадная колонна 13 3/8”, 9 5/8”

1 (одна) колонная головка со следующими характеристиками:

Давление согласно API: 5,000 фунт/кв. дюйм

Температура согласно API: класс P-U (-29 / + 121 ° C)

Материал согласно API: класс DD

КСИ (класс спецификации изделия) согласно API: PSL 2

ЭТ (эксплуатационные требования) согласно API: PR 2

Многоступенчатый корпус

Корпус, пригодный для подвешивания и герметизации нескольких клиновых подвесок.

Устьевой переводник лифтовой колонны (не прим.)

Это кольцо с фланцами на концах , называемое также фланцем уплотнения, которое включает в себя нижнюю центральную задвижку

НОМИНАЛЬНОЕ РАБОЧЕЕ ДАВЛЕНИЕ

Это максимальная величина гарантированного давления, которое оборудование может безопасно контролировать.

РАБОЧЕЕ ДАВЛЕНИЕ

Рабочее давление - это величина давления, при котором должно быть использована фонтанная арматура скважины.

Производитель – Cameron Romania S.r.l. (подразделение Schlumberger)

ПЕРЕЧЕНЬ ПОСТАВЛЯЕМОГО ОБОРУДОВАНИЯ (КОМПЛЕКТ КОЛОННОЙ ГОЛОВКИ)

| № | Наименование | Ед. изм. | Количество |
| --- | --- | --- | --- |
| 1 | СЕКЦИЯ A ПОСАДОЧНЫЕ КОЛЬЦА ПОСАДОЧНОЕ КОЛЬЦО, 30" SLIP-LOCK БЫСТРОСЪЕМНОЕ СОЕДИНЕНИЕ X 4.00" ТОЛЩИНА | шт | 1 |
| 2 | ПОСАДОЧНОЕ КОЛЬЦО ДЛЯ 13-5/8 ТИП 'SSMC' КОМПАКТНАЯ КАТУШКА 25.88 Н/ДИАМ X 18.96 В/ДИАМX 6.2” ДЛИНА | шт | 1 |
| 3 | СЕКЦИЯ B СБОРКА «SPLIT COMPACT SOLIDRILL» ВЕРХНИЙ УРОВЕНЬ НИЖНИЙ КОРПУС, SPLIT COMPACT 'SOLIDRILL, 13-5/8 5000 PSI BX-160 ФЛАНЕЦ ВЕРХНИЙ X 13-3/8 SLIPLOCK НИЖНИЙ С ДВОЙНЫМ 'T' УПЛОТНИТЕЛЯМИ ДЛЯ 13-3/8 ОБС. КОЛОННЫ, С ДВУМЯ 2-1/16 API 5000 PSI R-24 ВЫХОД, С 2-1/16 API VR, ПОСАДОЧНЫЙ БУРТ ДЛЯ ПОСАДОЧНОГО КОЛЬЦА; MИН. ОТВЕРСТИЕ 12.35 ДЮЙМ ISO 10423 & API 6A, PSL 2, PR2, КЛАСС ТЕМП: P-U; КЛАСС МАТЕРИАЛА: DD | шт | 1 |
| 4 | ВЕРХНИЙ КОРПУС, SPLIT COMPACT SOLIDRILL; 13-5/8 НОМ 5000 BX-160 НИЖНИЙ НА ШПИЛЬКАХ x 13-5/8 НОМ 10000 (CF-13) FASTLOCK HUB ВЕРХНИЙ, С ДВУМЯ 2-1/16 API 10000 ВЫХОД, С 2-1/16 API VR, MИН. ОТВЕРСТВИЕ 13.00 ДЮЙМ API PSL2, PR2, КЛАСС ТЕМП: P-U; КЛАСС МАТЕРИАЛА: DD | шт | 1 |
| 5 | СБОРКА ШИБЕРНОЙ ЗАДВИЖКИ, МОДЕЛЬ РУЧНАЯ FLS, 2 1/16, 5000 PSI | шт | 2 |
| 6 | СБОРКА ШИБЕРНОЙ ЗАДВИЖКИ, МОДЕЛЬ РУЧНАЯ FLS, 2-1/16, 10,000 PSI, API ФЛАНЕЦ X ФЛАНЕЦ, API 6A, PSL-2, PR-2, КЛАСС ТЕМП: P-U; КЛАСС МАТЕРИАЛА: DD | шт | 4 |
| 7 | КОМПЛЕКТ ФЛАНЕЦ 2-1/16 API 5,000 X 2 IN API LP РЕЗЬБА. API 6A, PSL-2, PR-2, КЛАСС ТЕМП: P-U; КЛАСС МАТЕРИАЛА: DD | шт | 2 |
| 8 | ГЛУХАЯ ПРОБКА, КРУГЛАЯ 2" LP, C 1/2" LP ТЕСТОВЫЙ ПОРТ | шт | 2 |
| 9 | 1/2" NPT СПУСКНАЯ ПРОБКА С ЗАЩИТНЫМ КОЛПАКОМ. Материал корпуса: нерж. сталь, Grade 316 | шт | 4 |
| 10 | ИГОЛЬЧАТЫЙ КЛАПАН 1/2 M/F NPT 10000 PSI | шт | 4 |
| 11 | ДАТЧИК ДАВЛЕНИЯ, 5000 PSI, материал корпуса: нерж. сталь | шт | 2 |
| 12 | ШПИЛЬКИ С ПОЛНОЙ РЕЗЬБОЙ, 7/8-9 UNC-2A X 6.00 ДЛИНА | шт | 16 |
| 13 | УТОЛЬЩЕННАЯ ШЕСТИГРАННАЯ ГАЙКА, 7/8-9 UNC-2B ASME B18.2.2, ASTM A194/ASTM A194M, GRADE 2H | шт | 32 |
| 14 | УПЛОТНИТЕЛЬНОЕ КОЛЬЦО, API ТИП RX-24, ВОСЬМИУГОЛЬНОЕ, НЕРЖ. СТАЛЬ, ТИП 316 | шт | 4 |
| 15 | ПРОБКА ДЛЯ СНЯТИЯ ЗАДВИЖЕК 1-1/2” 11-1/2 TPI - 3/4 TPF 'VEE' РЕЗЬБА | шт | 2 |
| 16 | УПЛОТНИТЕЛЬНОЕ КОЛЬЦО, API ТИП BX-160, ВОСЬМИУГОЛЬНОЕ, НЕРЖ. СТАЛЬ, ТИП 316 | шт | 1 |
| 17 | САЛЬНИК, 13-5/8 CF-13S, AISI 316 SST XYLAN, API 6A КЛАСС МАТЕРИАЛА: AA,BB,CC,DD, EE,FF | шт | 1 |
| 18 | ГЛУХОЙ ФЛАНЕЦ 2-1/16 API 10M, С 1/2" NPT; API 6A, КЛАСС ТЕМП: P; PSL-2; КЛАСС МАТЕРИАЛА AA,BB,DD, ЕЕ | шт | 2 |
| 19 | СТАНДАРТНЫЙ ДАТЧИК ДАВЛЕНИЯ 0-700bar (10000PSI / 69MPa) | шт | 2 |
| 20 | ШПИЛЬКИ С ПОЛНОЙ РЕЗЬБОЙ 3/4 – 10 UNC-2A X 5.25 ДЛИНА | шт | 32 |
| 21 | УТОЛЬЩЕННАЯ ШЕСТИГРАННАЯ ГАЙКА, 3/4 – 10 UNC-2В | шт | 64 |
| 22 | УПЛОТНИТЕЛЬНОЕ КОЛЬЦО, API TИП BX-152, API 6A PSL 4 | шт | 6 |
| 23 | СЕКЦИЯ C 9.5/8" СБОРКА КЛИНОВОЙ ПОДВЕСКИ И УПЛОТНЕНИЯ (АВАРИЙНЫЙ ТИП) СБОРКА КЛИНОВОЙ ПОДВЕСКИ, СПЕЦ ВОРОНКА С SSMC КЛИНЬЯ, 13 X 9-5/8, API 6A, PSL-3; PR2, КЛАСС ТЕМП: U; КЛАСС МАТЕРИАЛА: DD; АВАРИЙНЫЙ ТИП SOLIDRILL | шт | 1 |
| 24 | СБОРКА АВАРИЙНОГО УПЛОТНИТЕЛЬНОГО УСТРОЙСТВА, TИП SOLIDRILL 13 НОМ X 9-5.8 ОБС КОЛОННА С ДВОЙНЫМ ‘LS’ И ‘T’ УПЛОТНИТЕЛЯМИ, C J-СЛОТАМИ, МИН ОТВ.: 8.8” | шт | 1 |
| 25 | СЕКЦИЯ D 9.5/8" СБОРКА МАНДРЕЛЬНОЙ ПОДВЕСКИ И УПЛОТНЕНИЯ КОРПУС МАНДРЕЛЬНОЙ ПОДВЕСКИ, TИП SOLIDRILL, 13-НОМ, С 9-5/8 47 PPF,TENARIS HYDRIL BLUE РЕЗЬБА BOX НИЖНЯЯ X 11.000-4TPI LH ВНУТРЕННЯЯ STUB ACME РЕЗЬБА ВЕРХНЯЯ; API 6A; К/Т:P,S,T,U,V; К/М:AA,DD; PSL-2; PR-2. | шт | 2 |
| 26 | СБОРКА УПЛОТНИТЕЛЬНОГО УСТРОЙСТВА, TИП SOLIDRILL, ДЛЯ 13-5/8 НОМ КОЛОННОЙ ПОДВЕСКИ, С ДВОЙНЫМИ ВЕРХНЯМИ 'LS' УПЛОТНИТЕЛЯМИ, С НИЖНИМ S -УПЛОТНИТЕЛЕМ, С J-СЛОТАМИ. ~ МИН ОТВ: 9.735 | шт | 2 |
| 27 | СЕКЦИЯ F ЗАГЛУШКА СБОРКА НКТ АДАПТЕРА 13-5/8 API 10K FASTLOCK CF-13 НИЖНИЙ X 3-1/16 API 10K BX-154 ШПИЛИЧНЫЙ ВЕРХНИЙ, С 7-3/4 CANH УПЛОТНИТЕЛЬ КЛАСС МАТЕРИАЛА: DD, КЛАСС ТЕМП: P-U, API 6A 21ST EDITION, PSL-3. МАТЕРИАЛ КОРПУСА: 4130 LAS 75KSI YS | шт | 1 |
| 28 | СБОРКА ШИБЕРНОЙ ЗАДВИЖКИ, МОДЕЛЬ РУЧНАЯ FLS, 3-1/16, 10,000 PSI, API ФЛАНЕЦ X ФЛАНЕЦ, КЛАСС ТЕМП: P+U, КЛАСС МАТ: DD, PSL-3, PR-2 | шт | 1 |
| 29 | КОМПЛЕКТ ФЛАНЕЦ, 3-1/16 API-10,000 BX-154 ФЛАНЕЦ С 1/2" NPT ПРОБКА | шт | 1 |
| 30 | УПЛОТНИТЕЛЬНОЕ КОЛЬЦО, API ТИП BX-154, ВОСЬМИУГОЛЬНОЕ | шт | 2 |
| 31 | ШПИЛЬКИ С ПОЛНОЙ РЕЗЬБОЙ, 1-8 UNC-2A X 6.75 ДЛИНА | шт | 8 |
| 32 | УТОЛЬЩЕННАЯ ШЕСТИГРАННАЯ ГАЙКА, 1-8 UNC-2B | шт | 16 |
| 33 | СЕКЦИЯ G ЗАПАСНЫЕ ЧАСТИ КОЛЬЦЕВОЕ УПЛОТНЕНИЕ, TИП T, ДЛЯ 13-3/8" ОБС КОЛОННЫ, 13.375" ДО 13.442" ДИАМ | шт | 2 |
| 34 | КОЛЬЦЕВОЕ УПЛОТНЕНИЕ, TИП T ДЛЯ 13-3/8" ОБС КОЛОННЫ, 13.442" ДО 13.509" ДИАМ | шт | 2 |
| 35 | КОЛЬЦЕВОЕ УПЛОТНЕНИЕ, TИП T ДЛЯ 13-3/8" ОБС КОЛОННЫ, 13.308" ДО 13.375" ДИАМ | шт | 2 |
| 36 | СТОПОРНОЕ КОЛЬЦО ДЛЯ 13-3/8 ОБС КОЛОННЫ | шт | 4 |
| 37 | УПЛОТНИТЕЛЬНОЕ КОЛЬЦО, API ТИП RX-24, ВОСЬМИУГОЛЬНОЕ | шт | 4 |
| 38 | УПЛОТНИТЕЛЬНОЕ КОЛЬЦО, API TИП BX-160, ВОСЬМИУГОЛЬНОЕ | шт | 1 |
| 39 | САЛЬНИК, 13-5/8 CF-13S, AISI 316 SST XYLAN, API 6A КЛАСС МАТЕРИАЛА: AA,BB,CC,DD, EE,FF | шт | 1 |
| 40 | УПЛОТНИТЕЛЬ, TИП 'LS', 13-НОМ, 12.82 НДИАМ X 11.89 ВДИАМ | шт | 2 |
| 41 | КОЛЬЦЕВОЕ УПЛОТНЕНИЕ, ТИП "T" ДЛЯ 9-5/8" ОБС КОЛОННЫ | шт | 2 |
| 42 | КОЛЬЦЕВОЕ УПЛОТНЕНИЕ, ТИП "T" ДЛЯ 9-5/8" ОБС КОЛОННЫ (+1/2%) | шт | 2 |
| 43 | КОЛЬЦЕВОЕ УПЛОТНЕНИЕ, ТИП "T" ДЛЯ 9-5/8" ОБС КОЛОННЫ (-1/2%) | шт | 2 |
| 44 | СТОПОРНОЕ КОЛЬЦО ДЛЯ 9-7/8" X 7/32" | шт | 4 |
| 45 | ШЕСТИГРАННАЯ ЗАГЛУШКА 1/4 NPT --316 SS. | шт | 4 |
| 46 | УПЛОТНИТЕЛЬНОЕ КОЛЬЦО, API ТИП BX-154 | шт | 1 |
| 47 | УПЛОТНИТЕЛЬНОЕ КОЛЬЦО, API TИП BX-152 | шт | 2 |
| 48 | СЕКЦИЯ H ИЗНОСОЗАЩИТНАЯ ВТУЛКА ИЗНОСОЗАЩИТНАЯ ВТУЛКА, НИЖНЯЯ, SOLIDrill, 13-5/8 НОМ, MИН ОТВ: 12.300 | шт | 1 |
| 49 | ИЗНОСОЗАЩИТНАЯ ВТУЛКА, ВЕРХНЯЯ, SOLIDrill, 13-5/8 НОМ, MИН ОТВ: 8.845. | шт | 1 |

ITEM 1 - WELL HEAD (NEW DISSAMBLED SET)

Wellhead Configuration 1

A wellhead composed by Multistage Housing.

Section A - with surface casing 13 3/8”, 9 5/8”

One (1) wellhead unit with following technical specifications

Pressure Class API: 5000 psi

Temperature Class API: P-U (-29 / + 121 C)

Material Class API: Class DD

PSL (Product Specification Level) API: PSL 2

PR (Performance Requirements) API: PR 2

Multistage Housing

Housing, suitable to suspend and seal multiple casing hangers.

Tubing Head Adapter (n/a)

It is a spool, called also Seal Flange, which incorporates the lower Master Valve.

RATED WORKING PRESSURE (Rated W.P.)

It is the maximum guaranteed pressure value that the equipment can safely control.

WORKING PRESSURE (W.P.)

The Working Pressure is the pressure value at which the well control equipment will be exposed.

Manufacturer – Cameron Romania S.r.l. (Schlumberger affiliate)

THE LIST OF SUPPLIED EQUIPMENT (WELLHEAD SET)

| Item | Name of Goods | Unit | QTY |
| --- | --- | --- | --- |
| 1 | SECTION A LANDING RINGS LANDING RING, 30" SLIP-LOCK CONNECTOR X 4.00" THICK MIN. BORE: 25.29” | ea | 1 |
| 2 | LANDING RING FOR 13-5/8 TYPE 'SSMC' COMPACT SPOOL 25.88 O/DIA X 18.96 I/DIA X 6.2” LONG | ea | 1 |
| 3 | SECTION B SPLIT COMPACT HOUSING ASSEMBLY TOP LEVEL SUB-ASSY, LOWER HOUSING, SPLIT COMPACT 'SOLIDRILL', 13-5/8 5000 PSI BX-160 FLANGE TOP X 13-3/8 INTEGRAL SLIPLOCK BOTTOM W/ DOUBLE 'T' SEALS FOR 13-3/8 54.5 lbs/ft CASING, WITH TWO 2-1/16 API 5000 PSI R-24 STUDDED OUTLETS, WITH 2-1/16 API VR, WITH LANDING SHOULDER FOR WELLHEAD LANDING RING; MIN BORE 12.35 IN, ISO 10423 & API 6A, PSL 2, PR2 TEMP CLASS: P-U MATL CLASS: DD | ea | 1 |
| 4 | SUB-ASSY, UPPER SPOOL SPLIT COMPACT SOLIDRILL; 13-5/8 NOM 5000 BX-160 STUDDED BOTTOM x 13-5/8 NOM 10000 (CF-13) FASTLOCK HUB TOP, W/ TWO 2-1/16 API 10000 STDD OUTLETS, W/ 2-1/16 API VR, MIN BORE 13.00 IN, API PSL2, PR2 MATL CLASS : DD, TEMP CLASS: P-U | ea | 1 |
| 5 | GATE VALVE ASSEMBLY, MANUAL, MODEL FLS, 2-1/16 BORE, 5,000 PSI, 2-1/16 | ea | 2 |
| 6 | GATE VALVE ASSEMBLY, MANUAL, MODEL FLS, 2-1/16 BORE, 10,000 PSI, 2-1/16 API FLANGE X FLANGE, API 6A, TEMPERATURE CLASS P+U, MATERIAL CLASS DD, PSL-2, PR-2 | ea | 4 |
| 7 | FLANGE-COMPANION 2-1/16 API 5,000 X 2 IN API LP THREAD. API 6A 21TH ED; T/C: K,; M/C: DD,; PSL 2 | ea | 2 |
| 8 | BULL PLUG, ROUND 2" LP, W/1/2" LP TEST PORT, API 6A 21TH ED ISO 10423, MATL'S CLASS DD | ea | 2 |
| 9 | 1/2" NPT Bleeder Plug with Thread Protector. Body/Stem Material: Stainless Steel, Grade 316 Ball Material: Tungsten Carbide For Low Temperature NACE Service. PGI P/N: BV10N4-CO | ea | 4 |
| 10 | VALVE, NEEDLE 1/2 M/F NPT 10000 PSI STL FOR HYDR/AIR STRAIGHT TYPE WITH HANDLE @ 45 DEG. | ea | 4 |
| 11 | GAUGE, PRESSURE FOR CHRISTMAS TREE, DIAL SIZE - 4-1/2 INCH DIA, TUBE AND SOCKET MATERIAL - 316 STAINLESS STEEL, CONNECTION SIZE - 1/2 INCH NPT, CONNECTION LOCATION - LOWER, STANDARD PRESSURE RANGE - 5000 PSI, CASE MATERIAL: STAINLESS STEEL, RING: BAYONET LOCK, WINDOW: GLASS, DIAL: ALUMINUM, WHITE BACKGROUND BLACK FIGURES AND INTERVALS, POINTER: MICROMETER ADJUSTABLE. | ea | 2 |
| 12 | STUD, CONTINUOUS, 7/8-9 UNC-2A X 6.00 INCH LONG, DIMENSIONS PER CAMERONBOM SPECIFICATION, ASME A320/A320M GRADE L7, ZINC PLATED PER ASTM B633FE/ZN 5, FINISH TYPE II, API 20E BSL1 | ea | 16 |
| 13 | NUT, HEAVY HEX, 7/8-9 UNC-2B PER ASME B18.2.2, ASTM A194/ASTM A194M, GRADE 2H, ZINC PLATED PER ASTM B633 FE/ZN 5, FINISH TYPE II, API 20E BOLTING SPECIFICATION LEVEL 1 | ea | 32 |
| 14 | RING GASKET, API TYPE RX-24, OCTAGONAL, STAINLESS STEEL, TYPE 316, NO COATING, MAXIMUM HARDNESS 83 HRB, API 6A/ISO 10423, | ea | 4 |
| 15 | VALVE REMOVAL PLUG 1-1/2 IN 11-1/2 TPI - 3/4 TPF 'VEE' TUBING THD, 2-1/16 2,000 - 10,000, (1.25 HEX), API 6A 21TH EDITION / ISO 10423 MATERIALS CLASS DD | ea | 2 |
| 16 | RING GASKET, API TYPE BX-160, OCTAGONAL, STAINLESS STEEL, TYPE 316, NO COATING, MAXIMUM HARDNESS 83 HRB, API 6A/ISO 10423 | ea | 1 |
| 17 | GASKET, 13-5/8 CF-13S, AISI 316 SST XYLAN COATED, FOR USE IN API 6A MATERIAL CLASS AA,BB,CC,DD, EE,FF APPLICATIONS | ea | 1 |
| 18 | BODY; FLANGE, RECESSED BLIND 2-1/16 API 10M, W/ 1/2" NPT; API 6A TEMP CLASS P; PSL-2; MAT'L CLASS AA,BB,DD,EE | ea | 2 |
| 19 | STANDARD GAUGE, PRESSURE 0-700bar (10000PSI / 69MPa), 1/2NPT MALE BTM CONNECTION, 100mm DIA, PAINTED CASE, 0-700bar SCALING, WITHOUT FILLING (NOT ANTIVIBRATING), AISI 316 PRESSURE ELEMENT, WORKING TEMP -40 TO 200 C DEG (API TEMP RATING: P-X) | ea | 2 |
| 20 | STUD, CONTINUOUS 3/4 – 10 UNC-2A X 5.25 INCH LONG | ea | 32 |
| 21 | NUT, HEAVY HEX, 3 /4 – 10 UNC-2B PER ASME B 18.2.2, ASTM A194/ASTM 194M, GRADE 2H | ea | 64 |
| 22 | RING GASKET, API TYPE BX-152, LOW C STL, PLATED, API 6A PSL 4 | ea | 6 |
| 23 | SECTION C 9.5/8" SLIP HANGER AND SEAL ASSEMBLY ASSY, SLIP HANGER, SPECIAL BOWL W/ SSMC SLIPS, 13 X 9-5/8 API 6A, PSL-3; PR2, T/C: U; M/C: DD F/ EMERGENCY TYPE SOLIDrill | ea | 1 |
| 24 | ASSEMBLY; EMERGENCY SEAL PACKOFF, TYPE SOLIDrill, 13 NOM X 9-5/8 OD CSG, W/ DOUBLE "LS" SEALS, W/ J-SLOTS, W/ DOUBLE "T" SEALS, ~ MIN BORE: 8.88. | ea | 1 |
| 25 | SECTION D 9.5/8" MANDREL HANGER AND SEAL ASSEMBLY BODY, MANDREL CASING HANGER, TYPE SOLIDrill, 13-NOM, W/ 9-5/8 47 PPF,TENARIS HYDRIL BLUE THD BOX BTM X 11.000-4TPI LH INTERNAL STUB ACME THD TOP; ~ MIN BORE: TBA.API 6A; T/C:P,S,T,U,V; M/C:AA,DD; PSL-2; PR-2. | ea | 2 |
| 26 | ASSEMBLY; SEAL PACKOFF, TYPE SOLIDrill, F/ 13-5/8 NOM SIZE CASING HANGER, W/ DOUBLE UPPER 'LS' SEAL, W/ LOWER S-SEAL, W/ J-SLOTS. ~ MIN BORE: 9.735 **NACE MATERIAL; EOL-985 SEALS | ea | 2 |
| 27 | SECTION F ABANDONMENT CAP ASSY TUBING HEAD ADAPTER 13-5/8 API 10K FASTLOCK CF-13 BTM X 3-1/16 API 10K BX-154 STUDDED TOP, W/ 7-3/4 CANH SEAL PREP BOTTOM, MAT CLASS: DD, TEMP CLASS: P-U, API 6A 21ST EDITION, PSL-3. BODY MATERIAL: 4130 LAS 75KSI YS | ea | 1 |
| 28 | GATE VALVE ASSEMBLY, MANUAL, MODEL FLS, 3-1/16 BORE, 10,000 PSI, 3-1/16 API FLANGE X FLANGE, API 6A, TEMPERATURE CLASS P+U, MATERIAL CLASS DD, PSL-3, PR-2 | ea | 1 |
| 29 | ASSY, FLANGE, RECESSED BLIND, 3-1/16 API-10,000 BX-154 FLG W/ 1/2" NPT PLUG. API 6A 21TH EDITION; ISO 10423; T/C: P+U; M/C: DD/EE; PSL 3, PR2. | ea | 1 |
| 30 | RING GASKET, API TYPE BX-154, OCTAGONAL, LOW CARBON STEEL, ZINC PLATED, MAXIMUM HARDNESS 68 HRB, API 6A/ISO 10423 | ea | 2 |
| 31 | STUD, CONTINUOUS, 1-8 UNC-2A X 6.75 INCH LONG, DIMENSIONS PER CAMERONBOM SPECIFICATION, ASME A193/A193M GRADE B7, ZINC PLATED PER ASTM B633FE/ZN 5, FINISH TYPE II, API 20E BSL1 | ea | 8 |
| 32 | NUT, HEAVY HEX, 1-8 UNC-2B PER ASME B18.2.2, ASTM A194/ASTM A194M, GRADE 2H, ZINC PLATED PER ASTM B633 FE/ZN 5, FINISH TYPE II, API 20E BOLTING SPECIFICATION LEVEL 1 | ea | 16 |
| 33 | SECTION G SPARE PARTS RING, PACKING, TYPE T, FOR 13-3/8" CASING, 13.375" TO 13.442" OD 70A DURO, OIL RESISTANT, LOW TEMPERATURE, 70 HRD, SYNTHETIC RUBBER SEAL COMPOUND, PSL 1-3, API API 6A, MAT'L CLASS AA, BB,CC,DD,EE,FF TEMP RATING K,L,P,R,S,T. | ea | 2 |
| 34 | RING, PACKING, TYPE T, FOR 13-3/8" CASING, 13.442" TO 13.509" OD 70A DURO, OIL RESISTANT, LOW TEMPERATURE, 70 HRD, SYNTHETIC RUBBER SEAL COMPOUND, PSL 1-3, API API 6A, MAT'L CLASS AA, BB,CC,DD, EE,FF TEMP RATING K,L,P,R,S,T. | ea | 2 |
| 35 | RING, PACKING, TYPE T, FOR 13-3/8" CASING, 13.308" TO 13.375" OD 70A DURO, OIL RESISTANT, LOW TEMPERATURE, 70 HRD, SYNTHETIC RUBBER SEAL COMPOUND, PSL 1-3, API API 6A, MAT'L CLASS AA, BB,CC,DD,EE,FF TEMP RATING K,L,P,R,S,T. | ea | 2 |
| 36 | RING RETAINER FOR 13-3/8 OD CASING *** MATERIAL: AISI 1070, ALTERNATIVE METERIAL: MIL-W-21425B TYPE 1*** | ea | 4 |
| 37 | RING GASKET, API TYPE RX-24, OCTAGONAL, STAINLESS STEEL, TYPE 316, NO COATING, MAXIMUM HARDNESS 83 HRB, API 6A/ISO 10423 | ea | 4 |
| 38 | RING GASKET, API TYPE BX-160, OCTAGONAL, STAINLESS STEEL, TYPE 316, NO COATING, MAXIMUM HARDNESS 83 HRB, API 6A/ISO 10423 | ea | 1 |
| 39 | GASKET, 13-5/8 CF-13S, AISI 316 SST XYLAN COATED, FOR USE IN API 6A MATERIAL CLASS AA,BB,CC,DD EE,FF- APPLICATIONS | ea | 1 |
| 40 | SEAL, TYPE 'LS', 13-NOM, 12.82 OD X 11.89 ID X .75 THICK, ARTIC NITRILE MS-1073, W/ CLOSED COIL 316 SST OR 302 SST SPRING. API 6A, PSL-3. | ea | 2 |
| 41 | RING, SEAL TYPE "T" FOR 9-5/8" CSG. (70 DURO OIL RESISTANT) PSL 1-3, 6A 20TH ED; P,S,T,U; AA, BB,CC,DD,EE,FF. | ea | 2 |
| 42 | RING, SEAL TYPE "T" FOR 9-5/8" CSG. (70 DURO OIL RESISTANT) PSL 1-3, 6A 20TH ED; P,S,T,U; AA, BB,CC,DD,EE,FF. | ea | 2 |
| 43 | RING, SEAL TYPE "T" FOR 9-5/8 CSG (-1/2% TO NOM) (70 DURO OIL RESISTANT) API 6A 20TH ED; T/C: P,R, S,T,U; M/C: AA,BB,CC,DD,EE,FF; PSL 1-3 | ea | 2 |
| 44 | RING,RETAINER 9-7/8" FREE DIA (ID) X 7/32" CROSS SECTION *** MATERIAL: AISI 1070, ALTERNATIVE METERIAL: MIL-W-21425B TYPE 1*** | ea | 4 |
| 45 | PIPE PLUG HEX HD 1/4 NPT --316 SS. | ea | 4 |
| 46 | RING GASKET, API TYPE BX-154, OCTAGONAL, LOW CARBON STEEL, ZINC PLATED, MAXIMUM HARDNESS 68 HRB, API 6A/ISO 10423 | ea | 1 |
| 47 | RING GASKET, API TYPE BX-152, LOW C STL, PLATED, API 6A PSL 4, | ea | 2 |
| 48 | СЕКЦИЯ H WEAR BUSHING ASSY; WEAR BUSHING, LOWER,TYPE SOLIDrill, 13-5/8 NOM, MIN BORE: 12.300. W/ LOCK PIN GROOVE | ea | 1 |
| 49 | ASSY; WEAR BUSHING, UPPER, TYPE SOLIDrill, 13-5/8 NOM, MIN BORE: 8.845. W/ LOCK PIN GROOVE | ea | 1 |

Позиция 2 -30” ОБСАДНАЯ ТРУБА

| Предмет | Диаметр |  | Марка | Толщина стенки |  |  |  | Вес |  | Тип соедине-ния |  | Диапазон длинн по API |  | Количество для покупки |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | мм | дюйм |  |  | мм |  | дюйм |  | Кг/м фунт на фут |  |  |  |  |  | Кол-во |  | Единица (м/шт) |  |
| Обсадная труба | 762 | 30 | Х-56 | 25.4 |  | 1 |  | 461,2 310,2 |  | XLC-S |  | 3 |  | 46,44 |  | м |  |

| Кондуктор 30” |  |
| --- | --- |
| Обсадная труба по стандарту API 5CT | 30” |
| Соединение | XLC-S |
| Имеет подьемную заглушку для поднятия на буровую площадку |  |

Производитель – Nippon Steel corporation (Япония)

ITEM 2 – 30” CAISNG

| Item | Outside diameter |  | Grade | Wall thickness |  |  |  | Weight |  | Connection type |  | API length range |  | Quantity to purchase |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | inch |  | mm |  | inch |  | kg/m, ppf |  |  |  |  |  |  |  | q-ty |  | Unit (meter) |  |
| Casing | 762 | 30 | Х-56 | 25.4 |  | 1 |  | 461,2 310,2 |  | XLC-S |  | 3 |  | 46,44 |  | m |  |

| 30" Conductor |  |
| --- | --- |
| Casing according to API 5CT standard | 30” |
| Connection | XLC-S |
| It has a lifting plug for lifting to the drilling site |  |

Manufacturer – Nippon Steel corporation (Japan)

Позиция 3,6 -ТЕХНИЧЕСКОЕ ОПИСАНИЕ ОБСАДНОЙ ТРУБЫ 13 3/8”  и МУФТЫ ДЛЯ ОБСАДНОЙ ТРУБЫ 13 3/8”

| Предмет | Диаметр | Марка | Толщина стенки |  | Вес | Тип соедине-ния | Диапазон длинн по API | Количество для покупки |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | дюйм |  | мм | дюйм | кг/м, фунт на фут |  |  | Кол-во | Единица (м/шт) |
| Обсадная труба по стандарту API 5CT | 13 3/8” | N-80 | 12.2 | 0.48 | 68 | BTC | 3 | 181,98 | м |
| Муфта для обсадной трубы 13 3/8" | 13 3/8” | N-80 | 12.2 | 0.48 |  | BTC ниппель х муфта |  | 7 | Шт |

Производитель –TOO  Kazakhstan Pipe Threaders (Tenaris)

ITEM 3,6 – 13 3/8” CAISNG & CASING COUPLING 13 3/8"

| Item | Outside diameter | Grade | Wall thickness |  | Weight | Connection type | API length range | Quantity to purchase |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | inch |  | mm | inch | kg/m, ppf |  |  | q-ty | Unit (meter/ea) |
| Casing API 5CT | 13 3/8” | N-80 | 12.2 | 0.48 | 68 | BTC | 3 | 181,98 | m |
| Casing coupling 13 3/8" | 13 3/8” | N-80 | 12.2 | 0.48 |  | BTC |  | 7 | ea |

Manufacturer – Kazakhstan Pipe Threaders LLP (Tenaris)

Позиция 4,5 -ТЕХНИЧЕСКОЕ ОПИСАНИЕ ОБСАДНОЙ ТРУБЫ 9 5/8”  и МУФТЫ ДЛЯ ОБСАДНОЙ ТРУБЫ 9 5/8”

| Предмет | Диаметр | Марка | Толщина стенки |  | Вес | Тип соедине-ния | Диапазон длинн по API | Количество для покупки |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | дюйм |  | мм | дюйм | кг/м, фунт на фут |  |  | Кол-во | Единица (м/шт) |
| Обсадная труба по стандарту API 5CT | 9 5/8” | L-80 | 11.9 | 0.47 | 47 | TSH BLUE DOPELESS | 3 | 188,36 | м |
| Муфта для обсадной трубы 9 5/8" | 9 5/8” | L-80 | 11.9 | 0.47 |  | Премиум ниппель х муфта |  | 9 | Шт |

Производитель –TOO  Kazakhstan Pipe Threaders (Tenaris)

ITEM 4,5 – 9 5/8” CAISNG & CASING COUPLING 9 5/8"

| Item | Outside diameter | Grade | Wall thickness |  | Weight | Connection type | API length range | Quantity to purchase |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | inch |  | mm | inch | kg/m, ppf |  |  | q-ty | Unit (meter/ea) |
| Casing API 5CT casing | 9 5/8” | L-80 | 11.9 | 0.47 | 47 | TSH BLUE DOPELESS | 3 | 188,36 | m |
| Casing coupling 9 5/8" | 9 5/8” | L-80 | 11.9 | 0.47 |  | Премиум ниппель х муфта |  | 9 | ea |

Manufacturer – Kazakhstan Pipe Threaders LLP (Tenaris)